# GYDFH-100 FTTH Sat TV Optical Receiver with WDM

Solution Homologated with Tuves HD/DTH

GYDFH-100 is 1550nm satellite TV RF FTTH optical receiver with 1310nm/1490nm GPON WDM (upgradable to 1270nm/1577nm supporting XGPON ONU), receiving 1550nm Terrestrial TV and Sat RF optical signal from Passive Optical Network (PON) while passing 1310nm/1490nm or 1270nm/1577nm digital signals to ONU device. Working with GOYATECH dCSS satellite TV fiber optic transmitter or 1550nm fiber optic transmitter, GYDFH-100outputs high quality terrestrial TV and satellite TV RF.

## Features:

- Compact plastic flame retarding housing

- High Linearity Photodiode for Terrestrial TV and Satellite TV RF

- 88MHz~2350MHz (downstream) RF Output

- Optical AGC range: -10dBm ~ -2dBm

- 1310nm/1490nm Optical Bypass Port to ONU

- WDM can be upgraded to include 1270nm/1577nm reflection port for XGPON ONU

- DC Power and optical input LED indicator

- 12V DC power adapter

Specifications

| Item | Parameter |
| --- | --- |
| Input Optical Wavelength | 1540~1565 nm |
| Reflecting Channel Wavelength | 1260nm~1360nm and 1480nm~1500nm |
| Working Optical Power | -14dBm ~ -2dBm@1550nm |
| Optical Connector | SC/APC |
| Optical Return Loss | 45 dB |
| RF Bandwidth | 88 ~ 2350 MHz |
| RF Flatness | ± 4.0dB |
| RF Return Loss | >10 dB |
| Satellite User Bands | 32 (typical) |
| Each User Band bandwidth | 36MHz (typical) |
| RF Connector | American F Female |
| RF Output Impedance | 75Ω |
| AGC Range | -10dBm ~ -2dBm Optical Input |
| RF Output Level | >74dBμV @ (-10dBm~-2dBm) |
| Output to ONU |  |
| Loss at 1310nm/1490nm | <0.8dB |
| Isolation (Com-Pass at 1310nm) | > 35dB |
| Isolation (Com-Pass at 1490nm) | > 35dB |
| Isolation (Com-Reflect at 1550nm) | > 18dB |
| Output Connector | SC/UPC |
| Optical Return Loss | >45dB |
| Environment |  |
| Power Supply | 12 V 0.5mA (UL or CE approved) |
| Power Consumption | <2W (not including power adapter) |
| Working Temperature | -10 ~ +50℃ |
| Board Dimension (L*W*H) | 106mm×62mm×25mm |
| Weight (without power adapter) | 200g |

## Ordering Information

GYDFH-100-KX  FTTH Satellite RF Optical Receiver with WDM

X(WDM Option): None_GPON WDM only, X_ both GPON and XGPON WDM

=