# GYR1200 Universal Optical Node

GYR1200 Optical Node has universal die-cast aluminum housing outputting forward path analog TV, DVB-C and CMTS DS signals and sending upstream cable modem signals at regular or burst mode over the single Bi-directional fiber or the 2nd fiber. The forward path RF output can be 108dBµV/112 dBµV, single port or dual ports,  1005MHz or 1218MHz forward path bandwidth.  GYR1200  can support DOCSIS2.0, DOCSIS3.0 or DOCSIS3.1.

## Features:

- Aluminum Die Cast Universal Housing

- Two fibers or Single fiber bi-directional optical transmission

- 1005MHz or 1218MHz forward path RF bandwidth

- Single or Dual 108dBµV/112 dBµV Forward RF outputs

- Forward Path 15dB Slop and 15dB attenuator

- AGC effective at -5dBm~+1dBm optical input

- 5~85MHz/204MHz return RF bandwidth option

- Up to 16 CWDM DFB laser wavelength working at burst mode

- 4KV Surge Protection

- 60V or 220V power supply

## Specifications

| Item | Parameter |
| --- | --- |
| Forward Path Receiver |  |
| Forward RF Optical Wavelength | 1540~1560 nm |
| Working Optical Input Power | -7dBm ~ +2dBm |
| Optical AGC range | -5dBm ~ +1dBm |
| Forward Optical Power Test Point | 1V/mW |
| Fiber Input Connector | SC/APC |
| Optical Return Loss | 50 dB |
| RF Output Ports | 1 or 2 |
| RF Output Level | 108 dBµV or 112dBuV |
| RF Diplexer | 85/110MHz or 204/258MHz |
| RF Slop | 0~15dB adjustable |
| RF Attenuator | 0~15dB adjustable |
| RF Connector | American F |
| RF Output Impedance | 75Ω |
| Surge Protection at RF port | 4KV |
| RF Flatness | ± 1.5 dB (0dB Slop or after 15dB slop cable) |
| RF Return Loss | >16 dB |
| CNR | >44 dB @ -5dBm |
| CSO | <-60 dBc @77ch NTSC@0dBm |
| CTB | <-60 dBc @77ch NTSC@0dBm |
| Return Path Transmitter |  |
| Return Path Laser Wavelength | 1270nm, 1290nm, 1310nm, 1330nm, 1350nm, 1370nm, 1390nm, 1410nm, 1430nm, 1450nm, 1470nm, 1490nm, 1510mm,1530nm, 1590nm, 1610nm |
| Optical Output Power | 3±1 dBm |
| Laser Power Test Point | 1V/mW |
| Optical Indicator ON | >-3dBm |
| Return Path Laser off power | <-30dBm |
| Return Laser Rise/Fall Time | <1.5μs as defined by SCTE-174-2010 |
| Return RF Bandwidth | 5~85MHz/204MHz (5~30MHz filter on request) |
| Return RF Input Level | 17dBmV |
| Return RF Return Loss | 16dB |
| Return RF Flatness | ± 0.75 dB |
| Environment |  |
| Working Temperature | -20 ~ +55℃ |
| Storage Temperature | -40 ~ +85℃ |
| Power Supply | 60V or 220V AC |
| Power Consumption | 15~30W |
| Dimension (L*W*H) | 173mm×149mm×41mm |
| Weight | 3Kg |

## Ordering Information

GYR1200 -ABCDEFG Fiber to the Building two-way optical Node

A (RF Output Port): 1_1 RF output, 2_2 RF outputs

B (Forward Bandwidth): 1_1005MH z, 2_1218MHz

C (Frequency Duplexer): 1_85/110MHz, 2_204/258MHz

D (RF Output Level): 1_108 dBµV, 2_112 dBµV

E (Fiber Port): 1_Single fiber bi-direction 2_2 separate fibers

F (Burst Mode): 1_No Burst Mode, 2_Burst Mode

G (CWDM wavelength): 29_1290nm, 31_1310nm, 33_1330nm, 35_1350nm, 37_1370nm

39_1390nm, 41_1410nm, 43_1430nm, 45_1450nm, 47_1470nm

49_1490nm, 61_1610nm

H (Power Supply) 60_60V AC, 220_220V AC